The handset.exe program simulates a TMX or CF1000 handset. It

defaults to COM1. The com port number may be specified as an

argument e.g.: handset 2

To use this program you must construct an RS-232 to TTL level

translator. On the radio's DE-9S, pin 1 is TTL data in, pin 6 is TTL

data out and pins 8 and 9 are ground.

Note that this program must be running before the radio is powered

up such that the radio can detect a "handset" on its serial port.

The handset program is not pretty, but it should get the job

done if you don't happen to have a real handset.

Keyboard translation:

Due to the lack of similarity between the two layouts the following

conventions must be observed:

TMX              Keyboard

Up/Down          u/d

CL               c

E                Shift

RCL              r

STO              s

*                a      (Asterisk)

#                o      (Octothorpe)

0-9              0-9    (don't use keypad when shifting)

CL-*             Ctrl-R    (reset command)